# Physical Piano — Build Spec

Physical 11-key piano for the BCI arm/hand piano task. Replaces the toy mockup already validated on the physical arm/hand interface (11 keys, 4.5 cm wide with 0.5 cm gaps, one key-step = 5.0 cm, confirmed working).

The layout, key count, and note mapping intentionally match the BCI2000 simulator so the physical instrument behaves and sounds like `Piano_Application_vel.py`.

## Geometry (confirmed against hardware)

| Property | Value |
| --- | --- |
| Keys | 11, white, in a row |
| Key width | 4.5 cm (1.77″) |
| Gap between keys | 0.5 cm (0.20″) |
| Key length (depth) | 14 cm (5.5″) |
| Key pitch (center→center) | 5.0 cm (1.97″) — equals the arm's one key-step |
| Total keyboard width | 11 × 4.5 cm + 10 × 0.5 cm = 54.5 cm (21.5″) (+ frame margin) |

The 5.0 cm pitch is the load-bearing number: each physical key center must sit exactly 5.0 cm from the next (4.5 cm key + 0.5 cm gap) so the arm's key-step lines up. This pitch is ~1.97″, which is why the earlier "2-inch key-step" calibration still worked — only the key-width/gap split was mis-specified.

### Black keys (decorative)

Per supervisor: include the black keys and make them pressable, even though the task never uses them. Geometry mirrors the simulator exactly (Piano_Application_vel.py:775) so the physical board matches the screen:

| Property | Value |
| --- | --- |
| Count / positions | 7, between white keys at indices [0, 2, 3, 5, 6, 7, 9] |
| Width | 0.58 × key width = 26.1 mm |
| Length | 0.62 × key length = 86.8 mm, occupying the rear |
| Rise above white key tops | 10 mm (parametric) |
| Function | switched — one MX switch each, wired like the white keys |

Indices 2,3,5,6,7 are the real C♯/D♯/F♯/G♯/A♯ of the C–B octave; 0 and 9 continue the visual pattern across the padding keys, exactly as the sim draws it.

✅ Clearance confirmed. Black keys fill the rear 86.8 mm, so the fingers must contact the white keys within the front 53.2 mm (front_clear). Measured on the rig: the fingertip lands 35 mm from the front edge — well inside the clear zone, so no black key is in the finger's path and no repositioning is needed. This sets contact_x = 35 in the CAD, which also fixes where the switch and stop shoulders sit. (The CAD still warns if contact_x is ever moved into the black-key zone.)

### Black keys are switched, and that reshapes the white keys

Making the black keys pressable forced a change to a part that was otherwise settled. An MX switch housing is 15.6 mm across and the gap between white keys is 5 mm — the switch is three times too wide to fit, and even a bare pushrod carrying a keycap mount needs about 6 mm. There is no arrangement that puts a switch under a black key without widening that gap.

So each white key is notched by 7.5 mm on the side where a black key sits, opening the gap to 20 mm. This is exactly what a real piano does and for the same reason, so the board also ends up looking more authentic:

| White key | Rear width |
| --- | --- |
| no black neighbour | 45.0 mm |
| one black neighbour | 37.5 mm |
| two black neighbours | 30.0 mm |

The notch runs from the start of the black key zone to x = 122 mm, stopping short of the hinge knuckle, which needs its full 45 mm to locate the key on the pin.

The black key itself is a keycap pushed straight onto an MX stem, the way any keyboard keycap mounts — no hinge, no glue, no separate spring. It travels straight down while the white keys pivot. That difference is deliberate: a lever here would collide with the hinge brackets sharing the same gap, and black keys are never touched by the robot.

They also need no stop shoulders. The white keys have them because the robot presses with 362–520 gf and would otherwise hammer the switch. Black keys are hand-pressed, so bottoming out at 3.6 mm is ordinary keyboard use, well inside the switch's 80 M cycle rating.

⚠️ Three clearances are tight and should be checked on the prototype: the socket boss leaves 0.5 mm each side in the widened gap; a fully pressed black key clears the hinge posts by 0.9 mm; and the notch ends 0.5 mm before the knuckle. All positive, none generous.

## Key → note → pin mapping

18 keys: 11 white + 7 black. Key ids run left → right, id 0 being the leftmost key — the end carrying the orientation notch on the board.

White keys use the simulator's own frequencies unchanged (Piano_Application_vel.py:753), so the physical piano and the on-screen one sound identical.

| id | Key | Note | Hz | Pin |
| --- | --- | --- | --- | --- |
| 0 | white 0 (pad) | Bb3 | 233.08 | D2 |
| 1 | white 1 (pad) | B3 | 246.94 | D3 |
| 2 | white 2 | C4 | 261.63 | D4 |
| 3 | white 3 | D4 | 293.66 | D5 |
| 4 | white 4 | E4 | 329.63 | D6 |
| 5 | white 5 | F4 | 349.23 | D7 |
| 6 | white 6 | G4 | 392.00 | D8 |
| 7 | white 7 | A4 | 440.00 | D9 |
| 8 | white 8 | B4 | 493.88 | D10 |
| 9 | white 9 (pad) | C5 | 523.25 | D11 |
| 10 | white 10 (pad) | Db5 | 554.36 | D12 |
| 11 | black in gap 0/1 | A3 † | 220.00 | A5 |
| 12 | black in gap 2/3 | C♯4 | 277.18 | A0 |
| 13 | black in gap 3/4 | D♯4 | 311.13 | A1 |
| 14 | black in gap 5/6 | F♯4 | 369.99 | A2 |
| 15 | black in gap 6/7 | G♯4 | 415.30 | A3 |
| 16 | black in gap 7/8 | A♯4 | 466.16 | A4 |
| 17 | black in gap 9/10 | D5 † | 587.33 | D13 |

† Five black keys land on a real sharp. The outer two cannot: the simulator's padding keys are already a semitone apart (Bb3–B3 and C5–Db5), so no note exists between them. Those two continue the scale outwards instead. They are decorative keys the task never presses — change the values freely in pc/piano_listener.py, or set one to 0 to make that key silent.

Why D13 carries a padding black key. On many Uno clones pin 13 drives the onboard LED directly, which can load INPUT_PULLUP enough to read unreliably. It is therefore assigned the least important key, so a flaky pin costs nothing that matters. If D13 misbehaves, that one decorative key stops working and nothing else changes.

## Wiring

Every switch uses the Arduino's internal pull-up (INPUT_PULLUP), so no external resistors and no polarity to get right. Each switch goes between its pin and GND:

D2 ─────[ white key 0 ]───── GND

D3 ─────[ white key 1 ]───── GND

⋮             ⋮               ⋮

D12 ─────[ white key 10 ]──── GND

A5 ─────[ black gap 0/1 ]─── GND

A0 ─────[ black gap 2/3 ]─── GND

⋮             ⋮               ⋮

D13 ─────[ black gap 9/10 ]── GND     (all commons tie to one GND bus)

18 signal wires plus one common ground. A pressed key reads LOW; the firmware debounces for 15 ms, which clears the switch's 10 ms end-of-life bounce spec.

There is no speaker on the board. D13 was the only free pin and it is now a key — see below.

## Sound: played by the computer

The Arduino is a pure key scanner. It reports P <id> / R <id> over USB serial and makes no sound; pc/piano_listener.py receives those events and plays the notes.

This is not a workaround. The BCI2000 simulator already plays its notes through psychopy.sound, so PC audio is the existing architecture rather than a departure from it — and it satisfies "electronic sound, tunable on a computer" more literally than square waves ever did.

What it buys us

•  The pin budget closes exactly. 18 switches need all 18 usable pins on an Uno (D2–D13 plus A0–A5, with D0/D1 reserved for the serial link). A speaker would have needed a 19th.

•  Real timbre, polyphony, and any tuning — all in software, no reflashing.

•  Simpler firmware: no tone(), no frequency table, no EEPROM.

What it costs

•  The piano cannot make a sound standalone; the PC must be running the listener. Since BCI2000 is running during any real use, this is not a practical restriction.

•  Latency is roughly 15–40 ms end to end (USB serial plus audio buffer). For scale, a real piano action takes ~30 ms from key to hammer.

### Running it

pip install pyserial numpy pygame

python pc/piano_listener.py --test          # play every note, no hardware

python pc/piano_listener.py --port COM5     # normal use

--test is the quickest way to confirm audio works before any switch is wired. Add --echo to print each key event as it arrives.

The board also answers ping (replies PONG) and keys (lists every id with its pin) on the same serial port, which is handy for checking wiring one key at a time with the Arduino IDE's Serial Monitor.

## Bill of materials (buy)

| Qty | Item | Notes |
| --- | --- | --- |
| 1 | Arduino Uno or Nano | Uno is easiest to prototype; Nano if space-tight |
| 18 (+spares) | MX-style linear keyboard switches (~45 gf, 3.6 mm travel) — chosen | 11 white + 7 black. Kailh/Cherry linear red. Supplies contact + return spring + travel in one part, so there is no separate return spring. 45 gf actuation is far below the weakest finger (362 gf). Standard 14 × 14 mm plate mount. Wire the two switch pins to the Arduino pin/GND |
| ~ | Hookup wire, breadboard or proto-PCB | 18 signal + common GND |
| 1 | USB cable | Power, serial link, and the sound path |

No speaker, buzzer or amplifier is needed — the computer plays the notes. See Sound above. A piezo is still useful for bench-testing switches before the PC listener is set up, but it is not part of the finished instrument.

## Make (3D print / machine shop)

•  Key caps + hinge mechanism (11×, print): 4.5 cm × 14 cm tops with a consistent, repeatable press throw so the robot's fixed flex always closes the switch. Printing gives identical keys and lets you tune the throw. Important — protect the switch from overshoot: the finger drives ~3 cm but the switch actuates in ≤ a few mm, so give each key a hard bottom stop (key cap bottoming against the frame) positioned so the switch actuates just before the stop. The frame — not the switch internals — then absorbs the finger's 362–520 gf overshoot, which is what makes the mechanism durable. Parametric CAD implementing all of this (rear pin hinge, plunger, switch cradle, stop pads, return spring, up-stop hook) is in `../cad/piano_keys.scad` — defaults are self-consistent with the measured numbers. Set `contact_x` and the switch dimensions, then print ONE key + base cell to tune before batching 11.

•  Base / frame (print or machine shop): holds keys on 5.0 cm centers (4.5 cm key + 0.5 cm gap), mounts the switches under each key, and sits at the arm's working height. Total ~54.5 cm (21.5″) wide; confirm it fits inside the arm's reachable travel before cutting. Must be tiled: TechSpark's printers have a 254 mm build volume, so the 545 mm keybed prints as 3 tiles (5 + 5 + 1 cells) and is joined — the CAD's "tile" mode generates these.

•  Black keys (7×, print): decorative risers that drop into locating grooves at the white-key boundaries and are glued. No switches, no wiring.

## Purchasing & fabrication

•  Parts to buy, with links and budget: `PURCHASING.md`

•  Machine-shop / 3D-print plan (CMU TechSpark): `TECHSPARK_PLAN.md`

## Firmware

`../firmware/piano_firmware.ino`. Flash with the Arduino IDE. Pitches default to the simulator's frequencies and are tuned live over serial (see Sound above); the serial monitor at 115200 also prints press N / release N for bring-up testing.

## Press characterization (for switch selection)

The press is position (PD) control, not force control (PressFinger, RockScissorsPaper.cpp:127): the knuckle drives press_flexion = 0.80 rad and holds for PRESS_HOLD_MS = 400 ms (myAllegroHand.cpp:228). Contact force is therefore emergent (Kp ≈ 800–900 × position error into the surface, up to the torque cap), so it must be measured, not read from code.

| Spec | Value |
| --- | --- |
| Fingertip vertical stroke (resting → full press) | ~3 cm (measured, press-and-hold) |
| Resting fingertip height above table | ~4 cm (measured) |
| Pressed fingertip height above table | < 1 cm (measured) |
| Press force — index (left) finger | 362 g ≈ 3.55 N (measured) |
| Press force — middle (center) finger | 500 g ≈ 4.90 N (measured) |
| Press force — ring (right) finger | 520 g ≈ 5.10 N (measured) |
| Weakest finger (sets switch spec) | index, 362 g ≈ 3.55 N |

How measured: with a zeroed digital scale centered under one fingertip at key height, each finger was pressed and held using the bench hold mode in the hand controller (1/2/3 = press-and-hold left/center/right, L = lift; see myAllegroHand.cpp MainLoop), reading the settled value. N ≈ g × 0.0098.

The stroke/height numbers were read off the rig with the finger held in resting vs. pressed pose (press-and-hold bench mode, not the 400 ms flash). The stroke is the fingertip's own vertical descent, not the depth a key gets depressed.

Design implications:

•  Pre-travel is not a constraint. With ~3 cm of fingertip stroke, any switch's actuation travel (sub-mm) fits with room to spare, and the keys can have a real, piano-like key-cap travel (~0.5–1 cm) with the switch underneath.

•  Force sets the switch choice. The switch must fully actuate under the weakest finger, 362 gf. The chosen MX linear switch actuates at ~45 gf — an ~8× margin, so every finger triggers it reliably.

•  The switch must not absorb the overshoot. MX switches bottom out internally at 4 mm, but the finger keeps pushing with 362–520 gf (several times a human keystroke). The design therefore drives the stem only 3 mm and lands the key cap on integral stop shoulders in the socket, leaving 0.6 mm of unused switch travel even on a worst-case switch. The frame takes the load, not the switch. The CAD warns if these values are ever set inconsistently.

## How the key mechanism works

Each white key is a rear-hinged lever, like a real piano key. A small post (the plunger) on its underside rests directly on an MX switch stem:

finger presses here (35 mm from front)

↓

FRONT ┌─────────────────────────────────────┐ REAR

│              key lever              │

└──┬────────┬─────────────────────────┴──○ ← pivot (hinge pin)

│        │ plunger

hook     ┌─┴──┐

│stem│  ← MX switch: the spring inside pushes UP

│▓▓▓▓│    and is what holds the key up

════════════════╧════╧════════════════════════  base plate

Inside the switch, a spring-loaded stem slides down; a contact closes at ~2 mm of travel and the spring returns it. There is no separate return spring — the MX spring both holds the key up and returns it.

The hinge pin runs across the keyboard so the key pivots up and down. Nothing else locates the key: the pin holds the rear, and the stem's own top-of-travel limit sets the resting height (via the plunger length). There is no up-stop catch — it isn't needed.

### Why the key doesn't press its own switch

The MX spring's guaranteed return force is 15 gf minimum (datasheet RF), so nothing moves until something pushes harder than that. The lever geometry then divides the key's weight down:

| Key mass | 16 g, centre of mass 57 mm from the pivot |
| --- | --- |
| Plunger position | 92 mm from the pivot |
| Force at the stem | 16 × 57/92 = 9.9 gf |
| Guaranteed return force (datasheet RF, min) | 15 gf |
| Maximum safe key mass | 24 g — ours is 16 g |

By contrast the finger presses at the same 35 mm point, so its full 362 gf reaches the stem against a 45 gf requirement — a huge margin the right way.

⚠️ Print the keys at low infill (~20%) — this is now a hard limit. The datasheet guarantees only 15 gf of return force (RF, minimum), not the 30 gf preload previously assumed, so the maximum safe key mass is 24 g. A near-solid key at ~41 g would push 25.4 gf and would sag onto its switch and self-trigger. At ~20% infill (16 g → 9.9 gf) there is a 1.5× margin. The CAD warns if key_mass_g exceeds the limit.

## Keyboard height & arm integration

The toy mat modeled key width and length well, but as a flat mat (~1 cm above the table) it does not model a real key's thickness/travel. A real keyed instrument will be taller. This is not a problem because the hand+arm height is adjustable — the current mounting height is not fixed. So treat keyboard thickness as a free design parameter and match the arm height to it:

1.  The key/switch/hinge/stop stack is now fixed: key tops sit 42.5 mm above the table (30.5 mm of board on 12 mm feet). Call this K.

2.  Raise the arm so the resting fingertip hovers ~2–5 mm above the key tops (small positive clearance so a resting finger never holds a key down).

3.  The ~3 cm stroke then presses the key: only ~0.5–1 cm of key throw is needed to actuate + bottom out, so 3 cm ≫ clearance + throw (≈2 cm margin). The hard bottom stop absorbs the remaining overshoot.

To verify before finalizing: the arm's vertical adjustment range can reach K + clearance. Measured reference points on the current rig: resting fingertip ~4 cm above table, pressed fingertip < 1 cm above table (on the ~1 cm mat).

## Open items

•  (none blocking) — geometry, sound, and press force are all measured and the switch spec is set. Next physical steps: order the BOM, 3D-print the keys/hinge mechanism, bench-test firmware + tuner, then integrate under the arm.