# Physical Piano — Start Here

Building a real 11-key piano to replace the toy playmat in the BCI arm/hand piano task.

Status: design complete and verified against both the physical rig and the switch datasheet. Nothing has been bought or fabricated yet.

## Layout

docs/       specifications and plans (Markdown — the source of truth)

share/      the same docs as .docx, for the shared Drive folder

cad/        OpenSCAD models

firmware/   Arduino sketch — key scanner only, makes no sound

pc/         computer-side listener that plays the notes

tools/      scripts that generate share/*.docx from docs/*.md

### Documents

| File | What it's for |
| --- | --- |
| docs/SUPERVISOR_BRIEF.md | One page + the 4 decisions needed — for the supervisor meeting |
| docs/BUILD.md | The spec: dimensions, sound, measured forces, mechanism |
| docs/PURCHASING.md | What to buy, with links and budget (~$457) |
| docs/TECHSPARK_PLAN.md | Fabrication plan + parts manifest |
| docs/datasheets/ | Kailh switch datasheet — the source for every switch dimension |

### CAD — four files, split by role

| File | Contains | Used by |
| --- | --- | --- |
| cad/piano_params.scad | Single source of truth. Every shared dimension and all sanity checks. Edit here. | everything |
| cad/piano_keys.scad | Key levers + black keys — printed in both routes | both |
| cad/piano_base_printed.scad | All-printed base (cells, 3 tiles) | route A |
| cad/piano_base_laser.scad | Laser base (2 sheets + stop rail) + 12 printed brackets | route B |
| cad/piano_frame.scad | Cosmetic surround — cheeks and rails. Purely aesthetic, print last | either |

The two base_* files are alternatives — build one, not both. Either way you also need piano_keys.scad. Full breakdown in the TechSpark plan's parts manifest.

### Regenerating the .docx

node tools/md2docx.js docs/BUILD.md share/Physical_Piano_Build_Spec.docx

node tools/build_bom_docx.js          # the BOM, which has a custom layout

## What to do next, in order

### ① Right now (~1 hour, unblocks everything)

1.  Measure where the fingertips land on a key. ✅ 35 mm from the front edge, well inside the 53.2 mm clear zone — no black-key collision.

2.  Email TechSpark to book the consult — Jen Hitchcock, jhitchco@andrew.cmu.edu. Lead time, so start early.

3.  Order the electronics (~$100 — Arduino, MX switches, piezo, wire). They get bench-tested while the mechanical side is still in progress.

### ② Verify the CAD

1.  Install OpenSCAD (free) and open each of the four cad/*.scad files. The console prints the derived geometry and will warn if anything is inconsistent. Nothing has been rendered yet — confirm the shapes look right before committing to fabrication.

### ③ TechSpark

1.  Attend the consult — bring the questions in the TechSpark plan. The critical ones are laser bed size, 1.5 mm stock availability, and what laser time costs.

2.  Fabricate ONE key + base + black key and test it on the rig. Do not batch 11 until that passes.

### ④ Build

1.  Full run: 11 keys, 7 black keys, and the base for whichever route was chosen.

2.  Wire the 18 switches, flash the firmware, then run python pc/piano_listener.py --port COMx. Check audio first with --test, which needs no hardware.

3.  Raise the arm ~5–8 mm so resting fingertips hover 2–5 mm above the key tops.

4.  Re-run the demo sequence to validate.

## Two tracks run in parallel

•  Electronics — breadboard the Arduino, switches and piezo; test the firmware and tuner with no mechanical parts at all.

•  Mechanical — the slow track, gated by the TechSpark consult.

Start the electronics early so that half is proven before any key exists.