BCI ARM AND HAND PIANO TASK

Physical Piano — Parts to Purchase

Shopping list for the 11-key physical piano that replaces the toy playmat. Every dimension in the design has been measured against the robot on the bench.

| Parts to purchase ~$135 Sections 1–3, solder path | Fabrication $176–315 Billed by TechSpark | Keys 11 + 7 White, plus decorative black |
| --- | --- | --- |

Suggested first step:  order Sections 1–3 now. Electronics and mechanical fabrication are independent — with these parts on hand, the switches, sound, and tuning software can be built and tested on a breadboard while the keys are still being made at TechSpark.

## 01  Key Switches

MX-style linear switches. Each supplies the electrical contact, the return spring, and 4 mm of travel in one part. They actuate at ~45 gf — far below the robot’s weakest finger, measured at 362 gf.

| Item | Qty | Price | Source |
| --- | --- | --- | --- |
| Kailh linear red switches, 10-pack MX-compatible. Two packs covers 11 keys plus 9 spares. | 2 packs | $12 | Adafruit #4952 |
| Cherry MX Red 45g, 10-pack Equivalent alternative if Kailh is out of stock. | — | $24 | MechanicalKeyboards · Amazon |

## 02  Microcontroller and Sound

The Arduino reads the 11 keys and plays one tone per key. Pitches are set from a computer over USB and saved on the board.

| Item | Qty | Price | Source |
| --- | --- | --- | --- |
| Arduino Uno R3 Official board. | 1 | $28 | Arduino Store · SparkFun |
| Uno R3 compatible board Budget option, functionally identical. | — | $13 | ELEGOO |
| Piezo buzzer Driven directly by the Arduino; no amp needed. | 1 | $2 | Adafruit #160 |
| Enclosed piezo element Optional, louder than the bare buzzer. | 1 | $3 | Adafruit #1739 |

## 03  Wiring and Assembly

Bring up the firmware on a breadboard first (no soldering, fully reversible), then solder a permanent perfboard build — the keys are struck repeatedly at 362–520 gf, which can work a breadboard's connections loose over time.

| Item | Qty | Price | Source |
| --- | --- | --- | --- |
| Full-size solderless breadboard Bring-up and testing only. | 1 | $5 | Adafruit #239 |
| USB A–B cable Powers the Arduino; carries the tuning connection. Likely already in lab stock. | 1 | $6 | Adafruit #62 · Arduino Store |
| Hookup wire, 22 AWG stranded 11 signal lines plus a common ground, six colors. | 1 set | $15 | Adafruit #3111 · SparkFun |
| Perfboard Ground bus for the 11 switch commons — final build. | 1 | $11 | Adafruit #1606 |
| Beginner soldering iron kit Iron, stand, solder, sucker, wick. | 1 | $20 | Amazon |
| K&S 3 mm brass rod, 3-pack Hinge pins for the key levers — brass works as well as steel here and is easier to cut. | 1 | $8 | Amazon |
| M3 nut/bolt/washer assortment kit Screws and nuts for joining base sections. | 1 set | $12 | Amazon |
| Gorilla super glue (cyanoacrylate) Sets the black keys into their locating grooves. | 1 | $7 | Amazon |

No-solder alternative:  skip the perfboard and soldering kit above and use a screw-terminal shield instead — each wire clamps under a screw. Less permanent than solder but far more robust than a bare breadboard.  Amazon, ~$15

## 04  Fabrication — Not a Purchase

Keys, base, and black keys are made at CMU TechSpark and billed to a university account. PLA is $0.64/gram.

| Item | Qty | Price | Source |
| --- | --- | --- | --- |
| All parts 3D printed ~490 g. Base prints as three tiles — the printers are only 10″ wide — so the 50 mm key pitch must survive two joins. | — | $315 |  |
| Laser-cut base + printed keys ~275 g printed, plus laser time. Cuts all 11 switch holes in one operation on one sheet, with no tile seams. | — | $176 + time |  |

Which approach we use is the main question for the TechSpark consult. The case for laser cutting is accuracy — the arm is calibrated to that 50 mm pitch. On cost it is a modest win, not a dramatic one: laser machine time is billed hourly and is not published, so the net saving is likely $30–80 rather than half.

Prices are approximate and exclude tax and shipping; confirm at checkout. Full specification and fabrication plan are in the shared project folder alongside this document.